The GLink-3D IP family provides high speed die-on-die interface Master PHY and Slave PHY.  They are used to transmit data between dies, assembled using TSMC System on Integrated Chips (SoIC) 3D stacking technology (3DFabric). The IP supports both Wafer-on-wafer (WoW) and Chip-on-wafer (CoW) assembly, both face to face and face to back options. They are used to communicate between Master and Slave PHY. Both Master and Slave PHY are built of TX and RX Data and Command Slices in a modular way. Each Data Slice allows transferring 16 bits at bit rate 5.0 Gbps to one direction, totally 80 Gbps per Data Slice.

GUC leverages its multi-year GLink-3D experience to the development of UCIe-3D IP. GUC’s UCIe-3D IP follows the UCIe standard to provide industry-leading, 1.6Gbps to 2.2Gbps per lane die-to-die inter connect, achieving the best area efficiency of 20 Tbps/mm2 in full duplex. GLink-3D is used to transmit data between dies through TSMC System on Integrated Chips (SoIC-X) 3D stacking technology (3DFabric), which supports Chip-on-wafer (CoW) assembly, with face-to-face die stacking.

| GLink-3D / UCIe-3D |  |  |  |
| --- | --- | --- | --- |
| IP Licensing - Best PPA PHY and Controller - TX/RX module size, features, TSV/HB array customization - 3D and sub-system integration guideline | 3D stacking - 3D FE and BE design flows - Cross-die STA and power sign-off - D2D DFT | PI/SI and Thermal Co-Design - Top-die and bottom die co-design - Chip/PKG/System thermal co-simulation - Verification flow | Manufacturing and Production Management - 3D stacking & PKG Design - Qualification item, hardware and execution - Characterization, yield, DOE |

Die-on-Die (GLink-3D) IP

| Part_Number | Process | Description | Download |
| --- | --- | --- | --- |
| IGAD2DY02A | 5nm | GLink-3D Die on Die Interface PHY with 9 Tbps/mm2 |  |
| IGAD2DX03A | 7nm/6nm | GLink-3D Die on Die Interface PHY with 9 Tbps/mm2 |  |

Die-on-Die (UCIe-3D) IP

| Part_Number | Process | Description | Download |
| --- | --- | --- | --- |
| IGAD2DZ05A | 3nm | UCIe-3D Die on Die Interface PHY with 20 Tbps/mm2 |  |
| IGAD2DY10A | 5nm | UCIe-3D Die on Die Interface PHY with 20 Tbps/mm2 |  |